T H E   V I R T U A L   T C O D E   M A C H I N E

--   R E F E R E N C E   M A N U A L   --

COPYRIGHT (C) 1997,1998 NILS M. HOLM

1.     THE ARCHITECTURE..................................   2

2.     CALLING CONVENTIONS...............................   3

3.     INSTRUCTION CYCLES................................   5

4.     STARTUP CONDITIONS................................   5

5.     CONVENTIONS USED IN THE FOLLOWING SECTIONS........   6

6.     DECLARATIONS......................................   7

6.1.   THE FEATURE VECTOR................................   8

7.     LOAD AND STORE INSTRUCTIONS.......................   9

8.     ARITHMETIC INSTRUCTIONS...........................  10

9.     PROCEDURES AND FLOW CONTROL.......................  13

A.     OTHER INSTRUCTIONS................................  14

B.     SOURCE LEVEL DEBUGGING SUPPORT....................  15

C.     CREDITS...........................................  16

Tcode R.2 Language Reference

1#1.#. # T#TH#HE#E # A#AR#RC#CH#HI#IT#TE#EC#CT#TU#UR#RE#E

The  Tcode  machine  is  a virtual 16-bit machine basically

consisting of the following parts:

+---------------+FFFFh                   FFFFh+---------------+

|               |                             |     Stack     |

|               |                             |      and      |

|  U n u s e d  |       +-------------+  +--->|    Dynamic    |

|               |   +---|     IP      |  |    |    Storage    |

|               |   |   +-------------+  | +->|- - - - - - - -|

|- - - - - - - -|   |   |     RR      |  | |  |               |

|               |   |   +-------------+  | |  |    F r e e    |

|               |<--+   |     FP      |--+ |  |               |

| Tcode Program |       +-------------+    |  |  M e m o r y  |

|               |       |     SP      |----+  |               |

| Instructions  |       +-------------+       |- - - - - - - -|

|               |          REGISTERS          |    Static     |

|               |                             |     Data      |

|               |                             |               |

+---------------+0000h                   0000h+---------------+

CODE ARRAY                                    DATA ARRAY

There  are  two  byte-addressable memory regions called the

c#co#od#de#e  and  d#da#at#ta#a #  # a#ar#rr#ra#ay#y.  The  c#co#od#de#e # a#ar#rr#ra#ay#y holds the Tcode program

which  is  to be executed and the d#da#at#ta#a # a#ar#rr#ra#ay#y is used to hold the

data  used  by  the  program.  Each cell in one of the arrays is

completely  addressable  using  a 16-bit pointer. Therefore, the

maximum capacity per array is 65536 bytes.

Machine  words  --  which  are  always  2 bytes wide -- are

stored  with  the  least  significant  byte in the cell with the

higher address: 1234h = 34h 12h (little endian byte ordering).

The  Tcode  machine  has  four  16-bit wide special purpose

registers which are outlined in the following table.

R#Re#eg#gi#is#st#te#er#r #  #  #  #  #  #  #  # D#De#es#sc#cr#ri#ip#pt#ti#io#on#n

-#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#-

FP              Frame Pointer.

The frame pointer always points to the stack

frame (aka context) of the currently running

procedure. FP is implicitly used by the

instructions LDL, LDLV, SAVL, INCL which address

local objects. FP is modified only by HDR and

END instructions. See also CALLING CONVENTIONS.

- 2 -

Tcode R.2 Language Reference

IP              Instruction Pointer.

This register always points to the instruction

which will be interpreted next. IP is always

interpreted as an offset into the code array.

It cannot be accessed directly, but only through

jump, call, and branch instructions.

RR              Return Register.

This register is used to transport procedure

results back to the caller. The return register

is loaded by a POP instruction and saved by

CLEAN. See also CALLING CONVENTIONS.

SP              Stack Pointer.

The stack pointer points to the object most

recently placed on the stack. Moving an object

to the stack implicitly decreases SP by 2.

Removing an object increases it by 2.

SP may be explicitly modified using the STACK

instruction.

-#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#-

The  Tcode  machine  instructions  can be divided into four

groups:  _#d_#e_#c_#l_#a_#r_#a_#t_#i_#o_#n_#s,  _#l_#o_#a_#d_# _#a_#n_#d_# _#s_#t_#o_#r_#e  instructions,  _#b_#r_#a_#n_#c_#h_#e_#s,

and _#a_#r_#i_#t_#h_#m_#e_#t_#i_#c instructions.

D#De#ec#cl#la#ar#ra#at#ti#io#on#ns#s  will  be processed only once (therefore, they

may  be  resolved  in  a preprocessing step). This means that an

instruction like

S#ST#TR#R # 5#5 # H#H # e#e # l#l # l#l # o#o

will   not   create  a  new  string  literal  each  time  it  is

interpreted,  but  only at the first time. (One also might think

of  this  behaviour  as  creating  _#t_#h_#e_# _# _#s_#a_#m_#e_# _# _#o_#b_#j_#e_#c_#t each time a

declaration executed).

A#Ar#ri#it#th#hm#me#et#ti#ic#c #  # i#in#ns#st#tr#ru#uc#ct#ti#io#on#ns#s  expect  their  arguments  on  the

runtime  stack  and  also place their results there. Since there

are  no general purpose registers, most operations are performed

on stack elements.

2#2.#. # C#CA#AL#LL#LI#IN#NG#G # C#CO#ON#NV#VE#EN#NT#TI#IO#ON#NS#S

A  procdure  should  always  begin  with  a H#HD#DR#R instruction

which  saves  the caller's context and creates a new stack frame

and  end  with an E#EN#ND#D instruction which restores the saved stack

frame and jumps back to the caller.

- 3 -

Tcode R.2 Language Reference

A procedure call

P#P(#(a#a,#, # b#b,#, # c#c)#)

where _#a, _#b and _#c be global variables, is coded as follows:

L#LD#DG#G # a#a # L#LD#DG#G # b#b # L#LD#DG#G # c#c # C#CA#AL#LL#L # P#P # C#CL#LE#EA#AN#N # 3#3 # .#.

Each  L#LD#DG#G  instruction loads the value of a global variable onto

the  stack.  C#CA#AL#LL#L # P#P  performs  the  procedure call which returns

with  its  result  in  the  return  register R#RR#R. The final C#CL#LE#EA#AN#N

instruction  removes  the  three  arguments  from  the stack and

replaces  them  with  the  value  returned in R#RR#R so that the top

stack element finally holds the procedure return value.

Each  called  procedure  may  expect  the  following  stack

configuration:

FP+M -- Argument #1

FP+6 -- Argument #N-1

FP+4 -- Argument #N

FP+2 -- Return Address (saved by CALL or CALR)

FP+0 -- Old SP (saved by HDR)

FP-2 -- Local variable #1

FP-4 -- Local variable #2

FP-J -- Local variable #K

SP   -- ( Free Memory below )

Note  that  the arguments are passed to the procedure in reverse

order  with  the  first  argument  at the highest address. Both,

arguments   and  local  variables  may  be  accessed  using  L#LD#DL#L

instructions. Given the above context,

L#LD#DL#L # M#M

would access the first argument.

L#LD#DL#L # 4#4

always  loads  the  value  of  the  last argument, if any. Local

storage is accessed using negative offets:

N#NU#UM#M # 2#25#5 # S#SA#AV#VL#L # -#-4#4

would load the second local variable with the value 25.

- 4 -

Tcode R.2 Language Reference

3#3.#. # I#IN#NS#ST#TR#RU#UC#CT#TI#IO#ON#N # C#CY#YC#CL#LE#ES#S

A  cycle  is  the  period  which is required to execute one

single  Tcode  instruction. Each cycle consists of the following

steps:

1#1)#) Load the instruction pointed to by I#IP#P.

2#2)#) Increment I#IP#P by 1.

3#3)#)  If the instruction has an operand (indicated by bit #7 set),

load  the  machine  word  pointed  to  by  I#IP#P  into  an internal

register and increment I#IP#P by 2.

4#4)#)  If the loaded instruction is an I#IN#NC#CG#G, I#IN#NC#CL#L, or I#IN#NI#IT#T command,

load  another  machine  word  (pointed  to  by  I#IP#P) into another

internal register and increment I#IP#P by 2.

5#5)#)  If  the  loaded  instruction is valid, execute it, otherwise

signal an error and halt the machine.

These  steps  are  repeated  until  the  Tcode  machine  is

halted.

If  an  instruction  modifies stack elements, first all its

operands  are removed from the stack, then the operation denoted

by  the  instruction  is  performed,  and  finally the result is

placed on the stack.

Declaration  instructions may have more than two arguments.

See the DECLARATIONS section for details.

4#4.#. # S#ST#TA#AR#RT#TU#UP#P # C#CO#ON#ND#DI#IT#TI#IO#ON#NS#S

This  sections  describes  the  state  of the Tcode machine

when it is started.

The contents of the c#co#od#de#e # a#ar#rr#ra#ay#y and d#da#at#ta#a # a#ar#rr#ra#ay#y are undefined.

The content of the Return Registers R#RR#R is undefined.

The  Instruction Pointer I#IP#P is set to zero. (Hence, it is a good

idea to load programs at the bottom of the code array.)

The  Stack  Pointer  S#SP#P and the Frame Pointer F#FP#P are both set to

zero  (or  65536  on  32-bit hosts) so that decrementing them by

- 5 -

Tcode R.2 Language Reference

two  will  give  the  address of the highest addressable machine

word (65534).

The  internal  Extension Jump Table (see EXEC) of the machine is

initialized   with  the  addresses  of  implementation-dependant

procedures.

5#5.#. # C#CO#ON#NV#VE#EN#NT#TI#IO#ON#NS#S # U#US#SE#ED#D # I#IN#N # T#TH#HE#E # F#FO#OL#LL#LO#OW#WI#IN#NG#G # S#SE#EC#CT#TI#IO#ON#NS#S

S#Sy#ym#mb#bo#ol#l #  #  #  #  #  #  #  #  #  # D#De#en#no#ot#te#es#s # .#..#..#.

-#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#-

M, N            generic numeric values (16 bits wide).

.N		an unsigned numeric value. For example, if N=-2,

then .N=65534. Normally, the most significant

bit of each machine word is interpreted as a

sign flag (1 indicates a negative number). The

leading dot indicates that the MSB should be

treated as part of the value instead of a sign

indicator.

L               a label which may tag either a data word or a

procedure. Label IDs are 16 bits wide.

A               an address. An address is a offset into the

code or data array where the base (code or data

array) is implicitly determined by the

associated instruction. (16 bits wide.)

C               a character (an 8-bit wide value).

X1 ... XN       a vector containing N elements of the type X.

memory[X]       the content of the X'th machine word in the

data array.

memory::X       the content of the X'th byte in the data array.

S0 ... SN       the N+1 elements most recently pushed onto the

stack. When popping elements, S0 will be

removed first and SN will be removed last.

-#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#-

- 6 -

Tcode R.2 Language Reference

6#6.#. # D#DE#EC#CL#LA#AR#RA#AT#TI#IO#ON#NS#S

O#Op#pc#co#od#de#e # (#(h#he#ex#x)#),#,

I#In#ns#st#tr#ru#uc#ct#ti#io#on#n # F#Fo#or#rm#ma#at#t #  #  #  #  #  # D#De#es#sc#cr#ri#ip#pt#ti#io#on#n

-#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#-

81, CLAB L              Code LABel.

Define a label identified by the value L

which tags the following procedure.

85, CREF L              Code REFerence.

Define a word-size storage location

holding the address of the procedure

tagged by the label L.

84, DATA N              DATA definition.

Define a word-size storage location

containing the value N.

83, DECL N              vector DECLaration.

Define a vector with a length of N

machine words and undefined contents.

82, DLAB L              Data LABel.

Define a label identified by the value L

which tags the following data object.

86, DREF L              Data REFerence.

Define a word-size storage location

holding the address of the data object

tagged by the label L.

BF, EXT N C1 ... CN     EXTernal reference.

Create an EXTERNAL reference to the

symbol represented by the characters

C1 through CN at the current position in

the code array.

When interpreting a Tcode program, this

instruction should lead to an error. It

is limited to compiled programs.

89, INIT N V            INITialize.

Originally used to initialize the Tcode

environment -- hence its name.

Each program must begin with this

instruction. The argument N specifies

the Tcode version the program complies

to. This document describes version 2

of the Tcode language.

V is a bit vector describing some

- 7 -

Tcode R.2 Language Reference

features required by the following

program. See FEATURE VECTOR, below.

88, PSTR N C1 ... CN    define Packed STRing.

Define a vector with a length of

(N+2) / 2

machine words containing the characters

C1 through CN. Each character is stored

in a separate byte, not machine word.

All trailing bytes of the vector are

filled with zeroes so that a properly

terminated packed string is created.

BE, PUB N C1 ... CN     PUBlic reference.

Create a PUBLIC tag at the current

position in the code array.

The characters C1 through CN contain the

name of the tag which may be used for

external linkage.

When interpreting Tcode programs, this

instruction type may be ignored safely.

87, STR N M1 ... MN     define STRing.

Define a vector with a length of N+1

machine words containing the values

M1 through MN. The last vector member

always holds the value zero. The primary

use of this instruction is the creation

of string literals.

-#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#-

6#6.#.1#1 # T#TH#HE#E # F#FE#EA#AT#TU#UR#RE#E # V#VE#EC#CT#TO#OR#R

As   of  realease  2,  the  I#IN#NI#IT#T  instruction  contains  an

additional  argument  called  the  _#f_#e_#a_#t_#u_#r_#e_# _#v_#e_#c_#t_#o_#r. This is a bit

vector  with the size of a machine word describing some features

required  by the Tcode program containing the vector. Currently,

the following bits are used:

B#Bi#it#t # ### #  #  # V#Va#al#lu#ue#e #  #  # D#De#es#sc#cr#ri#ip#pt#ti#io#on#n # (#(i#if#f # b#bi#it#t # s#se#et#t)#)

-#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#-

0      1h     BASIC Extensions required.

1      2h     VIO (terminal I/O) Extensions required.

2      4h     Vector GRAPHICS Extensions required.

-#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#-

The  Tcode  interpreter  must  make  sure that all required

extensions  are  present  before  running  a  program containg a

feature  vector  with  any  bits  set.  If  it cannot provide an

extension   requested   by   the   program,  it  must  write  an

informational  message  describing  the  missing  extension  and

- 8 -

Tcode R.2 Language Reference

exit.  Although,  a  fully T3X compliant compiler may decide not

to  set  any  FV  bits, even if some of the above extensions are

used.

7#7.#. # L#LO#OA#AD#D # A#AN#ND#D # S#ST#TO#OR#RE#E # I#IN#NS#ST#TR#RU#UC#CT#TI#IO#ON#NS#S

O#Op#pc#co#od#de#e # (#(h#he#ex#x)#),#,

I#In#ns#st#tr#r.#. # F#Fm#mt#t.#. #  #  #  #  # D#De#es#sc#cr#ri#ip#pt#ti#io#on#n

-#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#-

15, DEREF       DEREFerence.

Remove two elements S0 and S1 and compute the

absolute address of the S0'th member of the

vector pointed to by S1:  S0*2 + S1 .  Push the

computed address.

When used together with IND, this instruction

dereferences the S0'th member of S1. Hence the

name of this instruction.

14, DREFB       DeREFerence Byte.

Remove two elements S0 and S1 and compute the

absolute address of the S0'th byte of the vector

pointed to by S1 (S0+S1). Push the computed

address. See also DEREF.

13, IND         INDirect access.

Replace the top element with the value stored in

the storage cell pointed to by the top element:

S0 := memory[S0/2] .

12, INDB        INDirect Byte access.

Replace the top element with a machine word

which contains the zero-extended byte located at

the address contained in the top element:

S0 := memory::S0 .

A8, LDG A       LoaD Global.

Push the value stored in the memory cell with

the address A:  memory[A/2] .

A9, LDGV A      LoaD Global Vector.

Push the address A.

AA, LDL N       LoaD Local.

Push the value stored at the N'th position

`below' the stack frame base. The address of

the cell is computed as follows:

FP - N*2

Consequently, negative values of N may be used

to access locations `above' the frame base.

- 9 -

Tcode R.2 Language Reference

AC, LDLAB L     LoaD LABel.

Push the address which is tagged by the label L.

Used to reference vectors and procedures.

AB, LDLV N      LoaD Local Vector.

Push the absolute address of a local object.

N specifies the offset of the object to the

stack frame base. The absolute address is

computed using the formula  FP - N*2 .

AD, NUM N       load NUMber.

Push the value N.

AE, SAVG A      SAVe Global.

Pop one element and save it in the memory cell

with the address A.

AF, SAVL N      SAVe Local.

Pop one element and save it in the storage cell

with the address  FP - N*2 .  See also LDL.

31, STORB       STORe Byte.

Pop two elements S0 and S1 and store the least

significant 8 bits of S0 in the byte pointed to

by S1:  memory::S1 := S0.

30, STORE       Pop two elements S0 and S1 and store the value

S0 in the memory cell pointed to by S1:

memory[S1/2] := S0.

-#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#-

8#8.#. # A#AR#RI#IT#TH#HM#ME#ET#TI#IC#C # I#IN#NS#ST#TR#RU#UC#CT#TI#IO#ON#NS#S

O#Op#pc#co#od#de#e # (#(h#he#ex#x)#),#,

I#In#ns#st#tr#r.#. # F#Fm#mt#t.#. #  #  #  #  # D#De#es#sc#cr#ri#ip#pt#ti#io#on#n

-#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#-

1B, ADD         Remove two elements S0 and S1 and push their

sum:  S1+S0 .

1D, BAND        Bitwise AND.

Remove two elements S0 and S1, perform a bitwise

AND on them and push the result:  S1 & S0 .

0E, BNOT        Bitwise NOT.

Invert each bit in the top element: ~S0.

1E, BOR         Bitwise OR.

Remove two elements S0 and S1, perform a bitwise

OR on them and push the result:  S1 | S0 .

- 10 -

Tcode R.2 Language Reference

20, BSHL        Bitwise SHift Left.

Remove two elements S0 and S1, shift the bits of

S1 to the left by S0 positions and push the

result: S1 << S0 .

21, BSHR        Bitwise SHift Right.

Remove two elements S0 and S1, shift the bits of

S1 to the right by S0 positions and push the

result: S1 >> S0 .

1F, BXOR        Bitwise eXclusive OR.

Remove two elements S0 and S1, perform a bitwise

XOR on them and push the result:  S1 ^ S0 .

19, DIV         integer DIVide.

Remove two elements S0 and S1, compute the

(signed) integer part of their quotient and

push it: S1 / S0 .

If S0=0, signal a fatal error and halt.

22, EQU         EQUal.

Remove two elements S0 and S1. Push TRUE, if

they are equal and otherwise FALSE:

S1=S0-> -1: 0 .

25, GRTR        GReaTeR than.

Remove two elements S0 and S1. Push TRUE, if S1

is greater than S0 and otherwise FALSE:

S1>S0-> -1: 0 .  S0 and S1 are both signed.

27, GTEQ        Greater Than or EQual to.

Remove two elements S0 and S1. Push TRUE, if S1

is greater than or equal to S0 and otherwise

FALSE:  S1>=S0-> -1: 0 .

S0 and S1 are both signed.

24, LESS        LESS than

Remove two elements S0 and S1. Push TRUE, if S1

is less than S0 and otherwise FALSE:

S1<S0-> -1: 0 .  S0 and S1 are both signed.

0D, LNOT        Logical NOT.

If the top element is equal to zero, replace it

with -1, otherwise with zero:  S0=0-> -1: 0 .

26, LTEQ        Less Than or EQual to.

Remove two elements S0 and S1. Push TRUE, if S1

is less than or equal to S0 and otherwise FALSE:

S1<=S0-> -1: 0 .  S0 and S1 are both signed.

1A, MOD         MODulo.

Remove two elements S0 and S1, compute their

division remainder and push it:  S1 MOD S0 .

- 11 -

Tcode R.2 Language Reference

S1 MOD S0 is defined as  S1 - S1/S0*S0  where

`/' denotes an integer division.

If S0=0, signal a fatal error and halt.

18, MUL         MULtiply.

Remove two elements S0 and S1, compute their

(signed) product and push it:  S1 * S0 . Do not

perform any overflow checking.

0C, NEG         NEGate.

Negate the top element: -S0.

23, NEQU        Not EQUal.

Remove two elements S0 and S1. Push TRUE, if

they are not equal and otherwise FALSE:

S1\=S0-> -1: 0 .

1C, SUB         SUBtract.

Remove two elements S0 and S1 and push their

difference:  S1-S0 .

41, UDIV	Unsigned integer DIVide.

Remove two elements S0 and S1, compute the

unsigned integer part of their quotient and

push it: .S1 / .S0 .

If S0=0, signal a fatal error and halt.

43, UGRTR       Unsigned GReaTeR than.

Remove two elements S0 and S1. Push TRUE, if

.S1 is greater than .S0 and otherwise FALSE:

.S1>.S0-> -1: 0 .

45, UGTEQ       Unsigned Greater Than or EQual to.

Remove two elements S0 and S1. Push TRUE, if

.S1 is greater than or equal to .S0 and

otherwise FALSE:  .S1>=.S0-> -1: 0 .

42, ULESS	Unsigned LESS than

Remove two elements S0 and S1. Push TRUE, if

.S1 is less than .S0 and otherwise FALSE:

.S1<.S0-> -1: 0 .

44, ULTEQ       Unsigned Less Than or EQual to.

Remove two elements S0 and S1. Push TRUE, if

.S1 is less than or equal to .S0 and otherwise

FALSE:  .S1<=.S0-> -1: 0 .

40, UMUL	Unsigned MULtiply

Remove two elements S0 and S1, compute their

unsigned product and push it:  .S1 * .S0 .

Do not perform any overflow checking.

-#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#-

- 12 -

Tcode R.2 Language Reference

9#9.#. # P#PR#RO#OC#CE#ED#DU#UR#RE#ES#S # A#AN#ND#D # F#FL#LO#OW#W # C#CO#ON#NT#TR#RO#OL#L

O#Op#pc#co#od#de#e # (#(h#he#ex#x)#),#,

I#In#ns#st#tr#r.#. # F#Fm#mt#t.#. #  #  #  #  # D#De#es#sc#cr#ri#ip#pt#ti#io#on#n

-#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#-

B6, BRF L       BRanch on False.

Remove one element S0 and branch to the

label L, if S0 is FALSE.

B7, BRT L       BRanch on True.

Remove one element S0 and branch to the

label L, if S0 is TRUE.

B3, CALL L      procedure CALL.

Push the current value of the instruction

pointer IP and then perform a branch to the

code array location tagged by the label L.

34, CALR        CALl through Register.

Push the current value of the instruction

pointer IP and then remove one element and

perform a branch to the location it points to.

The destination is implicitly located in the

code array of the program.

90, CLEAN N     CLEAN up arguments.

Remove N procedure arguments from the stack:

SP := SP + N*2

and then push the content of RR, the return

register.

0B, END         END procedure.

Remove two elements S0 and S1. Restore the

context of the calling procedure by loading FP

with S0 and then perform a branch to S1.

S1 is usually a return address which has been

saved by a CALL or CALR instruction.

B5, EXEC N      EXECute extension procedure.

Execute the extension procedure whose address is

stored in slot number N of the extension vector.

If N is not a valid slot number or points to

an unused slot, signal a fatal error and halt.

The exact semantics of EXEC depend on the called

procedure.

EXEC may be used to provide Tcode programs with

some extended functionality, like access to

resources of the host system (files, devices,

etc, ...).

BC, DNEXT L     Downward NEXT.

Remove two elements S0 and S1 and branch to the

label L, if  S1 <= S0 .  (*)

- 13 -

Tcode R.2 Language Reference

3D, HALT        Instantly halt the Tcode machine.

0A, HDR         HeaDeR.

Push the context of the calling procedure (FP)

and create a fresh procedure context by loading

FP with SP.

BA, JUMP L      Unconditionally jump to the label L.

B8, NBRF L      Nondestructive BRanch on False.

Branch to the label L, if S0 is FALSE. Do not

remove S0.

B9, NBRT L      Nondestructive BRanch on True.

Branch to the label L, if S0 is TRUE. Do not

remove S0.

0F, POP         Pop the top element S0 and load it into the

return register RR.

B2, STACK N     Move the stack pointer SP by N machine words:

SP := SP - N*2 .

Moving the stack pointer `down' (N>=1)

allocates space on the stack, moving it

`up' (N<=-1) deallocates space.

STACK is primarily used to allocate and release

dynamic memory in procedures.

BB, UNEXT L     Upward NEXT.

Remove two elements S0 and S1 and branch to the

label L, if  S1 >= S0 .  (*)

-#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#-

(#(*#*)#)  The  instructions  U#UN#NE#EX#XT#T  and D#DN#NE#EX#XT#T have been designed

for  use  in  counting loops (FOR-N#NE#EX#XT#T loops in BASIC). The idea

is  the  following:  At  the  end  of the loop, the current loop

index  and  the loop limit are both pushed onto the stack. U#UN#NE#EX#XT#T

compares   the   values   and  branches  out  of  the  loop,  if

_#i_#n_#d_#e_#x_#>_#=_#l_#i_#m_#i_#t.  D#DN#NE#EX#XT#T branches, if _#i_#n_#d_#e_#x_#<_#=_#l_#i_#m_#i_#t. Therefore, U#UN#NE#EX#XT#T

is  used in upward counting loops and D#DN#NE#EX#XT#T is used in countdown

loops.

1#10#0.#. # O#OT#TH#HE#ER#R # I#IN#NS#ST#TR#RU#UC#CT#TI#IO#ON#NS#S

O#Op#pc#co#od#de#e # (#(h#he#ex#x)#),#,

I#In#ns#st#tr#r.#. # F#Fm#mt#t.#. #  #  #  #  # D#De#es#sc#cr#ri#ip#pt#ti#io#on#n

-#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#-

46, DUP		DUPlicate

Push the current top of the stack (S0), thereby

- 14 -

Tcode R.2 Language Reference

duplicating it.

96, INCG A N    INCrement Global.

Add the value N to the value of the memory cell

located at the address A. This is exactly the

same as

LDG A NUM N ADD SAVG A

but more efficient.

97, INCL M N    INCrement Local.

Add the value N to the value of the memory cell

whose absolute address is  FP - M*2 .

INCL M N is equal to

LDL M NUM N ADD SAVL M

but more efficient.

11, NOP         No Operation.

Rest for a cycle.

47, SWAP	Exchange the values of S0 and S1.

-#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#-

1#11#1.#. # S#SO#OU#UR#RC#CE#E # L#LE#EV#VE#EL#L # D#DE#EB#BU#UG#GG#GI#IN#NG#G # S#SU#UP#PP#PO#OR#RT#T

O#Op#pc#co#od#de#e # (#(h#he#ex#x)#),#,

I#In#ns#st#tr#r.#. # F#Fm#mt#t.#. #  #  #  #  #  #  #  #  #  #  #  #  # D#De#es#sc#cr#ri#ip#pt#ti#io#on#n

-#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#-

C9, GSYM N C1...CN A	Name a global symbol. C1 through CN

contain the single characters of the

symbol name. `A' is the ID of the label

which marks the named symbol. GSYM

instructions should be generated only

for global variable names.

C8, LINE N		The following instructions have been

created from line N in the source code

this Tcode program has been created

from.

C9, LSYM N C1...CN A	Name a local symbol. C1 through CN

contain the single characters of the

symbol name. `A' is a signed number

holding the position of the variable

relative to the value of FP.

-#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#--#-

- 15 -

Tcode R.2 Language Reference

1#12#2.#. # C#CR#RE#ED#DI#IT#TS#S

This  document is part of the T3X Compiler which is subject

to the following terms.

T3X -- A Compiler for the Procedural Language T, version 3X

Copyright (C) 1996,1997,1998 Nils M. Holm.  All rights reserved.

Redistribution and use in source and binary forms, with or

without modification, are permitted provided that the following

conditions are met:

1. Redistributions of source code must retain the above

copyright notice, this list of conditions and the following

disclaimer.

2. Redistributions in binary form must reproduce the above

copyright notice, this list of conditions and the following

disclaimer in the documentation and/or other materials provided

with the distribution.

THIS SOFTWARE IS PROVIDED BY THE AUTHOR AND CONTRIBUTORS ``AS

IS'' AND ANY EXPRESS OR IMPLIED WARRANTIES, INCLUDING, BUT NOT

LIMITED TO, THE IMPLIED WARRANTIES OF MERCHANTABILITY AND

FITNESS FOR A PARTICULAR PURPOSE ARE DISCLAIMED. IN NO EVENT

SHALL THE AUTHOR OR CONTRIBUTORS BE LIABLE FOR ANY DIRECT,

INDIRECT, INCIDENTAL, SPECIAL, EXEMPLARY, OR CONSEQUENTIAL

DAMAGES (INCLUDING, BUT NOT LIMITED TO, PROCUREMENT OF

SUBSTITUTE GOODS OR SERVICES; LOSS OF USE, DATA, OR PROFITS; OR

BUSINESS INTERRUPTION) HOWEVER CAUSED AND ON ANY THEORY OF

LIABILITY, WHETHER IN CONTRACT, STRICT LIABILITY, OR TORT

(INCLUDING NEGLIGENCE OR OTHERWISE) ARISING IN ANY WAY OUT OF

THE USE OF THIS SOFTWARE, EVEN IF ADVISED OF THE POSSIBILITY OF

SUCH DAMAGE.

- 16 -