/****************************************************************************

** $Id: qt/doc/accel.doc   2.3.4-beta4   edited 2001-02-01 $

**

** Help with writing Qt/Embedded accelerated drivers

**

** Copyright (C) 1992-2000 Trolltech AS.  All rights reserved.

**

** This file is part of the Qt GUI Toolkit.

**

** This file may be distributed under the terms of the Q Public License

** as defined by Trolltech AS of Norway and appearing in the file

** LICENSE.QPL included in the packaging of this file.

**

** This file may be distributed and/or modified under the terms of the

** GNU General Public License version 2 as published by the Free Software

** Foundation and appearing in the file LICENSE.GPL included in the

** packaging of this file.

**

** Licensees holding valid Qt Enterprise Edition or Qt Professional Edition

** licenses may use this file in accordance with the Qt Commercial License

** Agreement provided with the Software.

**

** This file is provided AS IS with NO WARRANTY OF ANY KIND, INCLUDING THE

** WARRANTY OF DESIGN, MERCHANTABILITY AND FITNESS FOR A PARTICULAR PURPOSE.

**

** See http://www.trolltech.com/pricing.html or email sales@trolltech.com for

**   information about Qt Commercial License Agreements.

** See http://www.trolltech.com/qpl/ for QPL licensing information.

** See http://www.trolltech.com/gpl/ for GPL licensing information.

**

** Contact info@trolltech.com if any conditions of this licensing are

** not clear to you.

**

**********************************************************************/

/*! \page accel.html

\title Adding an accelerated graphics driver to Qt/Embedded

<p>

Qt/Embedded has the capacity to make use of hardware accelerators.

To do so for a PCI or AGP driver, you need to perform the following steps:

<p>

1) Define an accelerated descendant of QLinuxFbScreen. This

should implement QVoodooScreen::connect to map its registers.

Use qt_probe_bus to get a pointer to PCI config space. This is where

you should check that you're being pointed at the right device

(using PCI device/manufacturer ID information). Then use PCI config

space to locate your device's accelerator registers in physical memory

and mmap the appropriate region in from /dev/mem. There is no need to

map the framebuffer, QLinuxFbScreen will do this for you. Return FALSE

if a problem occurs at any point.

QVoodooScreen::initDevice will be called only by the QWS server and

is guaranteed to be called before any drawing is done (and so is a good place

to set registers to known states). connect will be called by every connecting

client.

<p>

2) Define an accelerated descendant of QGfxRaster. This is where the actual

drawing code goes. Anything not implemented by hardware can be passed

back to QGfxRaster to do in software. Use the optype variable to make

sure that accelerated and unaccelerated operations are synchronised

(if you start drawing via software into an area where the hardware accelerator

is still drawing then your drawing operations will appear to be in the

wrong order). optype is stored in shared memory and is set to 0 by

unaccelerated operations; accelerated operations should set it to 1.

When a software graphics operation is requested and optype is 1,

QGfxRaster::sync() is called; you should provide your own implementation

of this which waits for the graphics engine to go idle. lastop is also

available for optimisation and is stored in the shared space

- this will not be set by the software-only QGfx and can be used to store

the type of your last operation (e.g. drawing a rectangle) so that part

of the setup for the next operation can be avoided when a lot of the same

operations are performed in sequence.

All drawing operations should be protected via a QWSDisplay::grab()

before any registers, lastop or optype are accessed, and ungrabbed() at the

end. This prevents two applications trying to access the accelerator at once

and possibly locking up the machine. It's possible that your source

data is not on the graphics card so you should check in such cases and

fall back to software if necessary. Note that QGfxRaster supports some

features not directly supported by QPainter (for instance, alpha channels

in 32-bit data and stretchBlt's). These features are used by Qt;

stretchBlt speeds up QPixmap::xForm and drawPixmap into a transformed

QPainter, alpha channel acceleration is supported for 32-bit pixmaps.

<p>

3) If you wish, define an accelerated descendant of QScreenCursor.

restoreUnder,saveUnder,drawCursor and draw should be defined as

null operations. Implement set, move, show and hide. 4k is left for your

cursor at the end of the visible part of the framebuffer

(i.e. at (width*height*depth)/8 )

<p>

4) Implement initCursor and createGfx in your QScreen descendant.

Implement useOffscreen and return true if you can make use of offscreen

graphics memory.

<p>

5) Implement a small function qt_get_screen_mychip, which simply

returns a new QMychipScreen

<p>

6) Add your driver to the DriverTable table in qgfxraster_qws.cpp

- e.g.

{ "MyChip", qt_get_screen_mychip,1 },

<p>

The first parameter is the name used with QWS_DISPLAY to request your

accelerated driver.

<p>

7) To run with your new driver,

export QWS_DISPLAY=MyChip (optionally MyChip:/dev/fb<n> to request a

different Linux framebuffer than /dev/fb0)

run the program

<p>

If your driver is not PCI or AGP you'll need to inherit QScreen instead

of QLinuxFbScreen and implement similar functionality to QLinuxFbScreen,

but otherwise the process should be similar. The most complete example

driver is qgfxmach64_qws.cpp; qgfxvoodoo_qws.cpp may provide a smaller

and more easy to understand driver.

*/